Display control utilities for the 1400LT

The  included  programs  are display  control  utilities  for the

1400LT.  Included are a  display blanking program,  (which will either

turn off the backlight, or blank a CRT); small programs to turn on and

off  the backlight; and a small  program to return the current display

device  in the form  of an error  level, which can  be used to control

batch  file execution. Each  of these programs  is described in detail

below.

SB14.COM - Screen blanking utility

A number of complaints have been raised about the standby mode of

the  1400, especially  with regards  to communications  programs since

when the 1400 goes into standby mode it locks the communications port.

This  program allows the backlight for the LCD to be turned off during

periods  of user inactivity, which does not drop the power consumption

down  to standby  levels, but does  significantly reduce  it, and also

gets rid of the annoying whine the backlight produces. In addition, if

the 1400 is connected to an external CRT, the CRT is blanked to extend

the tube life. (This is especially helpful with the 1400 since when it

goes into standby mode, it switches to the CRT).

The  program can be  loaded to just  look at the  keyboard, or to

look  at both the keyboard and any video (INT10) calls to the BIOS. In

the  former case, regardless of what is  going on with the screen, the

program  will time out if there is no keyboard activity. In the latter

case,  it will only time out when there is no keyboard activity and no

BIOS  video I/O.  (Some programs write  directly to  video memory, and

only  rarely call the video  BIOS. SB14 has no  way of knowing what is

going on and can time out in these cases).

If  the screen  backlight is  off when  it is  time to  blank the

screen (the backlight can be turned off with the program BLOFF below),

when  SB14 goes to turn  the screen back on,  it realizes it shouldn't

and  will NOT power the backlight back  up, which is just as it should

be.

If invoked with no parameters, SB14 will load itself in a mode to

ignore  video I/O, and with a 60 second time delay. For information on

the call form for SB14, type SB14 ?, the following screen will result,

and SB14 will NOT be loaded:

�-----------------------------------------�

�  SB14 - Screen blanker [Version 2.00]   �

�  Copyright (C) Richard Haw  1988        �

�-----------------------------------------�

�                                         �

�   SB14 {switch} /tnnn                   �

�                                         �

�   Where {switch} is                     �

�                                         �

�        /v    for video monitor on       �

�         ?    for this help information  �

�                                         �

�   /tnnn sets the blank time to nnn      �

�         seconds (decimal).              �

�                                         �

�-----------------------------------------�

To  load  SB14 with  the video  mode enabled,  simply add  the /v

switch,  to set a time  other than the default  of 60 seconds, add the

/tnnn,  where nnn represents the number  of decimal seconds before the

screen will blank (the valid range for nnn is from 1 to 65535).

When loading, the program will return with a screen informing you

of the load options, for example if invoked as:

SB14 /v/t120

the following screen would be displayed:

�-----------------------------------------�

�  SB14 - Screen blanker [Version 2.00]   �

�  Copyright (C) Richard Haw  1988        �

�-----------------------------------------�

�                                         �

�   Keyboard monitor on                   �

�   Video monitor on                      �

�                                         �

�   Time delay set to   120 seconds       �

�                                         �

�-----------------------------------------�

The  program is of  the terminate and stay  resident type, and is

not  smart enough to avoid loading itself  more than once. If you load

it  more than  once, unexpected and  unwanted problems  can arise. For

this  reason it is suggested  that once you find  what delay times are

comfortable  for  you,  place  the  command  to  load  SB14  into your

autoexec.bat file.

BLOFF.COM - Backlight off

BLON.COM - Backlight on

These programs just turn the LCD backlight off or on, as the case

may  be.  Executing  either of  these  programs with  an  external CRT

enabled  does nothing. The backlight is turned off and on via a "brute

force"  approach, meaning the  system doesn't know  it has been turned

off, and any reset or change of the display mode (switching to the LCD

or  CRT using the ctrl-alt-F11 or ctrl-alt-F12 keys, or going into set

up will turn the backlight on).

VIDM.COM - Test display mode

This  program just looks at what display is currently active, and

returns an appropriate error code which can be used in a batch file to

control  execution. An error level of 0 is returned if an external CRT

is  in use, and an error level of  1 is returned if the LCD is active.

An  example batch file (which  just lists out which  display is on) is

included  as  VIDMODE.BAT. The  most useful  application of  this (the

motive  behind it actually) is using it  in the startup file to figure

out  which display  is active,  and use  the dos  MODE command  or SET

command  to set things up for  software which looks lousy in "emulated

CGA" on the LCD but looks good in BW mode.

Source for BLOFF/BLON/VIDM

The source code for BLOFF.COM BLON.COM and VIDM.COM are included,

(as  I said, a simple brute force approach is used). This approach can

be  used from any language which allows for access to the IO ports, an

example  basic  program  to  achieve the  same  thing  is  included as

SCREEN.BAS.

Notes and comments

These programs just sort off sprung up out of a need for the this

type  of feature on  my 1400LT, and  they seem to  work fine with what

programs  I use, which is of course not to say they will work with all

programs. Any comments or suggestions in the area of program conflicts

is  welcomed, but I can't  say as I will be  able to do anything about

it.

One of the obvious things to include in a screen blanker would be

a feature that allows mouse input to reset the blank timer. I have had

no  luck in this area with the 1400  so far. The mouse can be detected

in  two ways, either through the communications interrupt (0Ch) or the

mouse  interrupt (Microsoft compatible 33h). Unfortunately, the best I

have  come up with doesn't work  under very many differing conditions.

Any input in this area is more than welcome.